| Unit code | SISOCYT001 |
| --- | --- |
| Unit title | Set up, maintain and repair bicycles |
| Application | This unit describes the performance outcomes, skills and knowledge required to select, set up and maintain bikes used for recreational cycling activities. It covers: selecting and preparing bikes to suit participant needs and maintaining and undertaking basic repairs.; This unit suits individuals who assist with the delivery of recreational activities involving setting up, maintaining and repairing bicycles in the outdoor recreation industry. No occupational licensing, certification or specific legislative requirements apply to this unit at the time of publication. |
| Pre-requisite unit | Nil |
| Competency field |  |
| Unit sector | Outdoor Recreation |
| Elements | Performance criteria |
| 1. Select and set up bikes | 1.1 Identify, from activity plans, characteristics and needs of participants, activity terrain and route 1.2 Select bikes to meet needs, size and proportions of riders and activity conditions 1.3 Make required adjustments to ensure bikes are correct fit for riders and suitable for terrain requirements 1.4 Complete pre-departure safety and serviceability checks and action faults according to organisational procedures |
| 2. Maintain and repair bicycles | 2.1 Use personal protective equipment and safe manual handling techniques 2.2 Interpret and follow maintenance schedules and manufacturers instructions 2.3 Inspect bikes and identify repair and maintenance requirements 2.4 Clean bikes and parts using appropriate cleaning agents to prevent corrosion and deterioration 2.5 Complete maintenance on bicycle components to ensuring safe operation 2.6 Complete repairs according to reports or in response to incidents 2.7 Refer complex repair and maintenance requirements to technical specialists 2.8 Complete maintenance records according to organisational procedures 2.9 Assemble an in-field repair kit for use during activities |
| Foundation skills Reading skills to: - interpret factual activity, plan information and maintenance schedules in familiar formats - interpret potentially unfamiliar and complex manufacturer instructions and specifications Writing skills to: - complete basic documents requiring factual information including fault tags and maintenance records Numeracy skills to: - complete bike measurements to approximate best fit for participants - complete basic calculations involving ratios to dilute cleaning products - identify and calculate accurate bicycle system settings and adjustments from specifications Planning and organising skills to: - manage own timing to complete bike preparations, repairs and maintenance within departure deadlines Technology skills to: - use specialist bicycle tools and equipment |  |
| Range of conditions |  |
| Assessment Requirements |  |
| Performance evidence | Evidence of the ability to complete tasks outlined in elements and performance criteria of this unit in the context of the job role: for three different participants complete the following bike adjustments: - saddle position and height - handlebar height and width - distance from saddle to handlebars - position of gears and brakes complete pre-departure safety and serviceability checks including: - bells, reflectors and lights - handlebars and plugs - wheels, hubs and spokes - suspension - headsets - tyre condition and tyre pressure - chains and gears - brakes - bolts or quick release skewers clean and service the following bicycle systems: - frame - drivetrain - steering - brakes - suspension - wheels and hubs - tyres, including two puncture repairs - accessories remove, adjust and refit or replace two worn or damaged components: - lubricate components - complete maintenance ecords for each bike serviced |  |
| Knowledge evidence | Demonstrated knowledge required to complete the tasks outlined in elements and performance criteria of this unit: organisational procedures for completing repairs, maintenance, safety and serviceability checks different types of bikes used for recreational cycling activities, their differing construction features and how features are suited to different surfaces and terrain: - road bikes - mountain bikes - hybrid bikes - fixed gear how participant characteristics, cycling surfaces and terrain affect the selection of bikes: - participant characteristics: - age, height, reach and weight - current cycling experience and skill level - fitness level and physical capabilities - injuries and medical conditions - emotional, behavioural and intellectual ability or disability - cycling surface: - sealed roads, formed bike paths or trails - unsealed surfaces including gravel roads, fire trails, single track, open or rocky ground - gradient of terrain - manufacture specifications for bike use bike sizing guidelines, and how to take measurements to suit rider size for: - road bikes - mountain bikes - hybrid bikes types of adjustments made to bikes to suit participant characteristics, surface and terrain and how to make these: - saddle position and height - handlebar height and width - distance from saddle to handlebars - position of gears and brakes - suspension - tyre pressure including range of suitable pressures types of pre-departure safety and serviceability checks, how to complete these and what tools would be used, to include those for: - legally required accessories e.g. bells, reflectors, lights - handlebars and plugs - wheels, hubs and spokes - suspension - headsets - tyre condition and tyre pressure - chains and gears - brakes components and key operational features of these bicycle systems: - frame - drivetrain - steering - brakes - suspension - wheels and hubs - tyres - accessories for each of the above systems: - how to remove, adjust and refit components - how to identify worn or damaged components for replacement - cleaning requirements and techniques - lubrication requirements, as relevant types of repairs referred to specialist technicians for tyres, how to remove tubes, repair punctures and re-inflate different types of cleaning products and how these are used on different bike parts: - cleaning solutions and sprays for removing mud and dirt - degreasers for cleaning chains and other moving parts - brake cleaners - suspension solutions and sprays - rust prevention polishes and sprays types and use of tools and equipment used to maintain and repair bicycles including those typically used in the field spare parts typically carried in the field safe manual handling techniques for bending, lifting and carrying heavy equipment during maintenance tasks types and correct use of personal protection equipment for maintaining bicycles a range of formats for and inclusions of maintenance documents: - fault tags and reports - repair and maintenance reports. |  |
| Assessment conditions | Skills can be assessed in the field or in an operational depot or a combination, provided that the situation is appropriate to the repair or maintenance task. A depot can be: - an industry workplace or - a simulated industry environment set up for the purposes of assessment. Assessment must ensure use of: - personal protective equipment for repair and maintenance activities - bicycles of different types including: - road - mountain - hybrid - spare parts for all bicycle system components - tools used to maintain and repair bicycles: - Allen keys - cassette remover - chain breaker - chain whip - crank puller and crank wrench - pliers, cable cutting pliers - pressure gauge - pumps and connectors - puncture repair kit - screwdrivers - spanners: ring, open ended, shifting, two cone, pedal, headset, sockets and ratchets - spoke key - tyre levers - lubricants - cloths, stiff and soft brushes used for cleaning - cleaning products for bikes including: - agents for removing mud and dirt - degreasers - brake cleaners - suspension cleaners - rust prevention agents - participant profiles - activity plans - manufacturer’s cleaning, repair and maintenance instructions - service document templates - organisational: - procedures for completing repairs, safety and serviceability checks - maintenance schedules Assessors must satisfy the Standards for Registered Training Organisations requirements for assessors, and: - have a collective period of at least three years’ experience with an organisation providing recreational cycling programs, or as a bicycle technician, where they have applied the skills and knowledge covered in this unit of competency. |  |
| Unit mapping information | No equivalent unit |  |
| Links | Link to Companion Volume Implementation Guide. |  |